NAME :  K.Avinash

REG NO: (192412662)

COURSE NAME : 5G 6G COMMUNICTION (ECA5601)

DATE : 12/08/2026

| EX NO: 4 | OFDMA Resource Block Allocation |
| --- | --- |

AIM: To simulate and analyze OFDMA (Orthogonal Frequency Division Multiple Access) resource block allocation and evaluate system performance in terms of resource utilization and user throughput using MATLAB.

Procedure:

- Define the system bandwidth and calculate the number of available resource blocks (RBs) based on RB size (e.g., 180 kHz per RB).

- Identify the number of active users in the OFDMA system for resource sharing.

- Allocate resource blocks to each user based on a predefined or equal distribution strategy.

- Compute Resource Block Utilization (%) using the ratio of allocated RBs to total RBs.

- Assign modulation efficiency values (e.g., QPSK, 16-QAM, 64-QAM) for each user.

- Calculate User Throughput (Mbps) using allocated RBs and modulation efficiency.

- Plot graphs for RB allocation and utilization/throughput analysis using MATLAB.

- Analyze results to evaluate spectrum efficiency and system performance.

Objective 1 Matlab Code and Output: clc; clear; close all; % System parameters bandwidth = 10e6; % 10 MHz rb_size = 180e3; % 180 kHz per RB % Total Resource Blocks total_RB = floor(bandwidth / rb_size);

% Users

users = 1:6;

% RB allocation (simple equal + remainder distribution) base = floor(total_RB / length(users));

alloc = base * ones(1,length(users)); alloc(1:mod(total_RB,length(users))) = alloc(1:mod(total_RB,length(users))) + 1; utilization = (alloc / total_RB) * 100; disp('Total Resource Blocks:') disp(total_RB) disp('RB Allocation per User:') disp(alloc) figure

% -------- Graph 1: RB Allocation per User -------- subplot(2,1,1) stem(users, alloc, 'filled', 'LineWidth', 2) title('OFDMA Resource Block Allocation per User') xlabel('User Index') ylabel('Number of Resource Blocks') grid on

% -------- Graph 2: RB Utilization -------- subplot(2,1,2) plot(users, utilization, '-o', 'LineWidth', 2) title('OFDMA Resource Block Utilization (%)') xlabel('User Index') ylabel('Utilization (%)') grid on

OUTPUT:

Result:

The first graph shows how available resource blocks are distributed among multiple users in the OFDMA system.